Arcade Racing Simulators for use in VR Bays

Objective:

In the automotive industry, hydraulics make it easy for cars and trucks to steer, stop, and it also provides the regenerative power to boost fuel efficiency. Hydraulics are also used in a variety of applications across several industries including the stabilization of airplanes as well as large machinery in unlevel conditions. How can you use fluid power to stabilize the seat in a car?

Project Description

Teams of students will be tasked with designing a chair to simulate performance as seen in drivers ed or arcade racing. Repurposing a YouTube video from any video game, or pre-prepared map for biking or fitness that already has motion control output... (I.e. vector/speed/direction) a team of students will program the chair that can replicate the anticipated movements that would occur if there were to be a real person operating that car.

Start with the basics:

- Slow playback speed to create programming and speed up as you develop.

- Start with pre-sets for (downhill, uphill, left, right 30 degrees) start with basics and then differentiate those commands later.

- Sizing, flow and power output will greatly affect speed, smoothness and performance of the simulator

  - Research and perform sizing calculations before selecting a hydraulic power unit to ensure there is proper pressure available in the system for expected performance.

    - 12V or 24V AC or DC power supplies may not provide the power required for the project with a human occupant)

  - Design of cylinder angles to the chair are important for stability.

  - Management of the flow exchanged between cylinders and valving is critical for sizing

Materials:

Note: This project is anticipated to cost around $2,000. NFPA may be able to support with component donations or a mentor to assist with project development.

- Square metal frame. Teams can build their own or use 80/20 or a similar product.

- Connection points for cylinders.

- Bucket seat

- 4 cylinders with about 300 pounds in total capacity

  - 75-100 pounds/cylinder

  - 1" cylinder? Consideration for speed and force

  - Note: Cylinders should not be mounted parallel to each other

- Hydraulic Power Unit with 110VAC and built-in safety relief valve.

  - Dual pump with unloader (hi-lo)

- Valving:

  - Standard DO directional valve or cartridge valve technology.

  - 3 position/4 way valves.

  - solenoid with levers

  - Simple on/off: compensated valve, perhaps manual flow controls

- Controller

  - At least 8 PWM outputs

  - At least 4 digital inputs

  - Level sensor (zero point)

  - Inclinometers

  - HMI or just computer for programming

    - PLCs are the industry standard but schools can elect to use Raspberry PI or an equivalent.

  - Incorporation of an E-Stop Emergency Stop Switch

- Others?

Scoring Criteria (if applicable):

Does the timing match the video? Did the actions or movements perform as planned?

Advanced Options:

- Design with a 3-cylinder axis.

- Add cylinders with feedback and closed loop control.

- Consider restraint for load testing or adding a human occupant. This would require proper testing prior to adding a human occupant.

NFPA Competencies (if applicable):

- Understand fluid power benefits and limitations

- Conceptual and theoretical understanding of fluid power laws and principles (including energy transfer and power efficiency)

- Understand fluid power components and circuits

- Understand the impact of fluid properties, i.e., fluid viscosity, on fluid power system efficiency and performance.

- Understand machine level requirements and translate into fluid power system requirements

- Apply design, simulation and analysis tools to fluid power components and systems

- Appropriately size components in fluid power systems

- Integrate sensing and electronic control functions with fluid power components and systems

- Cite hands-on experience with fluid power components and systems

- Inspect, analyze and develop corrective action for product failure

Educational Outcomes:

Implementation:

What is the knowledge level? (Ex.High school or early college)

What is the cost per number of students/teams of students served?

How many class hours are needed to complete the project?